Purchase Specifications for a 
2-3+ Nautical Mile Range Marine Lantern

Overview

This specification is for a 2-3NM+ marine lantern.

1.0 Light Characteristics

The lanterns light source will be one (1) LED.

The lantern output shall be available in red, green, white, yellow and blue.

The lantern shall have a typical maximum available intensity of up to:

- Red lantern output 24.7cd

- Green lantern output 26.6cd

- White lantern output 29.2cd

- Yellow lantern output 18.8cd

The lantern shall have a visible range of 2-3NM+.

The lantern shall have a horizontal output of 0-360 degrees.

The lantern shall have a vertical divergence of 7 degrees.

The lantern shall have up to 256 IALA recommended user-adjustable flash characteristics.

The lantern shall have multiple intensity settings.

2.0 Electrical Characteristics

The lantern shall have integrated circuit protection.

The lantern shall have a nominal voltage of 3.6V DC.

The lantern shall have an autonomy >30 days.

The lantern shall have an operating temperature range between -40 to 80°C.

3.0 Solar Characteristics

The lantern shall have two (2) multicrystalline solar modules.

Each module shall be 1.25watts.

4.0 Power Supply

The battery type shall be Nickel Metal Hydride (NiMH).

The battery shall have a capacity of 8Ah.

5.0 Physical Characteristics

The body of the lantern shall be manufactured from UV-stabilised polycarbonate.

The lantern lens shall be manufactured from UV-stabilised polycarbonate or equivalent.

The lantern shall have a lens diameter of 150mm (5⅞ inches).

The lens design shall be single LED optic.

The lantern shall have a 200mm bolt pattern.

The lantern shall have a height of up to 240mm (9½ inches).

The lantern shall have a width of 231mm (7⅛ inches).

The lantern shall have a mass from 1.4kg (3⅛lbs) dependent on model.

The lantern shall have a life expectancy of up to 12 years.

6.0 Options

The lantern shall be offered with the following options available from the manufacturer:

- 16Ah Battery

- IR programmer

- GPS Synchronisation

- 50mm pole mount adapter plate

7.0 Certifications

The lantern shall be IP68 waterproof.

The manufacturer shall be ISO9001:2008 certified.

The lantern shall meet the following certifications ANSI C63.4-2009.  EN61000-6-3:2007.  EN61000-6-1:2007.  IEC61000-4-6:2008.  IEC61000-4-3:2010.  IEC61000-4-6:2008.  IEC61000-4-2:2008. MIL-STD-810F.  MIL-STD-202G

The lantern shall meet IALA recommendation E-200-1 Signal colours

8.0 Warranty

The lantern shall have a three (3) year warranty, excluding battery which will have a warranty of one (1) year.